PROJECT 10 – INTERNET OF THINGS (IOTs)

Smart Socket Project (IoT with Arduino + ESP8266)

🔹 1. Project Overview

This project demonstrates the concept of the Internet of Things (IoT).

🎯 Goal:

- Convert a normal household socket into a smart socket

- Control it remotely using a mobile phone over the internet

🔹 2. What is IoT?

Internet of Things (IoT) =
Controlling physical devices via the internet from anywhere in the world.

📌 Examples:

- Turning on a light remotely

- Controlling AC from another city

- Switching appliances ON/OFF via phone

🔹 3. Key Difference from Other Wireless Systems

| Technology | Range | Limitation |
| --- | --- | --- |
| Bluetooth | Short range | Same room |
| IR Remote | Line-of-sight | Very limited |
| IoT (Wi-Fi) | Global | Internet required |

👉 IoT allows control from anywhere in the world 🌍

🔹 4. Components Used

🧩 Hardware

- Arduino Uno

- ESP8266 (Wi-Fi module)

- Relay Module (5V, single channel)

- Triple socket (extension box)

- Jumper wires

- Breadboard

- LED + 220Ω resistor (for testing)

🔹 5. Core Concept of the System

🔁 Data Flow

📱 Mobile App → 🌐 Internet → ESP8266 → Arduino → Relay → Socket

🔹 6. Relay Module Explained

⚙️ Function:

- Acts as a switch controlled electronically

🔌 Control Side (Low Voltage)

- VCC → 5V

- GND → GND

- Signal → Arduino digital pin

⚡ Power Side (High Voltage)

| Pin | Meaning |
| --- | --- |
| COM | Common input |
| NO | Normally Open |
| NC | Normally Closed |

🧠 Relay Logic

- NO (Normally Open)
→ OFF by default
→ Turns ON when signal = HIGH

- NC (Normally Closed)
→ ON by default
→ Turns OFF when signal = HIGH

🔹 7. ⚠️ Safety Warning

You are working with 220V AC electricity ⚡

✔ Always:

- Insulate connections with electrical tape

- Avoid touching exposed wires

- Double-check wiring

🔹 8. Socket Wiring

🔌 Cable Colors (AC System)

| Color | Function |
| --- | --- |
| Brown | Live (Phase) |
| Blue | Neutral |
| Yellow-Green | Ground |

👉 Only Live (Brown) is connected through the relay

🔹 9. Arduino Connections

🔌 Relay → Arduino

| Relay Pin | Arduino |
| --- | --- |
| Signal | Pin 9 |
| VCC | 5V |
| GND | GND |

🔌 ESP8266 → Arduino

| ESP8266 Pin | Arduino |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| TX | RX (Pin 0) |
| RX | TX (Pin 1) |

⚠️ Important

- Remove RX/TX wires before uploading code

- Reconnect after upload

🔹 10. ESP8266 Role

The ESP8266:

- Connects Arduino to Wi-Fi

- Receives commands from internet

- Sends them to Arduino

🔹 11. Mobile App Used

📱 Blynk

Used to:

- Send ON/OFF commands

- Create a control interface

🔹 12. Blynk Setup Steps

🧱 Step-by-Step

- Install Blynk app

- Create account

- Create new project

- Select:

  - Board → Arduino Uno

  - Connection → Wi-Fi

- Get Auth Token (code sent to email)

🔹 13. App Interface Setup

- Add Button Widget

- Configure:

  - Pin → D9 (relay pin)

  - Mode → Switch

🔹 14. Code Logic (Simplified)

🧠 Steps

- Connect ESP8266 to Wi-Fi

- Authenticate using Blynk token

- Wait for mobile commands

- When button pressed:

  - HIGH → Relay ON → Socket ON

  - LOW → Relay OFF → Socket OFF

🔹 15. Testing Process

🧪 Step 1: LED Test

- Control LED via Blynk

🧪 Step 2: Relay Test

- Send HIGH/LOW manually

🧪 Step 3: Full System

- Control socket via phone

🔹 16. Final Result

✔ Socket turns ON/OFF remotely
✔ Controlled via internet
✔ Works from anywhere 🌍

🔹 17. Real-Life Applications

- Smart home systems

- Remote appliance control

- Energy management

- Safety (turn off iron remotely 🔥)

🔹 18. Possible Upgrades

- Use 4-channel relay → control multiple devices

- Add power monitoring

- Build full smart home system

- Use NodeMCU (ESP8266 standalone) instead of Arduino

🚀 Big Picture (For You)

This project is VERY important for your path:

You’re now touching:

- Embedded systems

- IoT infrastructure

- Smart energy control (ties directly to your power systems + solar goals)

👉 This is literally how smart grids and smart homes work at a basic level