PROJECT 11 - IoT Smart Lamp with Google Assistant

In this project, we extend the IoT smart socket system by adding voice control using Google Assistant.

👉 Goal:

- Turn a lamp ON/OFF using voice commands

- Control it from anywhere via the internet

🧠 Concept: How It Works

This project combines three main systems:

- Arduino + ESP8266

  - Connects the system to the internet

- Blynk App

  - Sends control signals to Arduino

- IFTTT + Google Assistant

  - Converts voice commands → internet commands

🔁 System Flow

Voice Command → Google Assistant → IFTTT → Blynk Server → ESP8266 → Arduino → Relay → Lamp ON/OFF

🔧 Components Used

- Arduino Uno

- ESP8266 WiFi Module

- Relay Module (5V)

- Lamp / Socket

- Jumper wires

- Breadboard

🔌 Circuit Connections

📡 ESP8266 → Arduino

- VCC → 3.3V

- GND → GND

- RX → TX (Pin 1)

- TX → RX (Pin 0)

🔌 Relay → Arduino

- Signal → Pin 9

- VCC → 5V

- GND → GND

⚠️ Be careful when working with 220V AC (mains electricity)
Always insulate connections properly.

📱 Step 1: Blynk Setup

- Open Blynk app

- Create new project → “Smart Lamp”

- Select:

  - Device: Arduino Uno

  - Connection: WiFi

- Copy Auth Token (sent via email)

Add Button Widget:

- Pin: D9

- Mode: Switch

- Labels:

  - ON → “Lamp ON”

  - OFF → “Lamp OFF”

💻 Step 2: Arduino Code (mBlock Logic)

- Initialize ESP8266 using:

  - WiFi name (SSID)

  - Password

  - Auth Token

- Relay logic:

  - HIGH → Lamp ON

  - LOW → Lamp OFF

🌐 Step 3: IFTTT Setup

Create Account → Go to “Create”

🟢 Applet 1: Turn ON Lamp

IF (Trigger):

- Service: Google Assistant

- Phrase: "Open lamp"

THEN (Action):

- Service: Webhooks

- URL:

http://blynk-cloud.com/YOUR_AUTH_TOKEN/update/D9

- Method: PUT

- Body:

["1"]

🔴 Applet 2: Turn OFF Lamp

IF (Trigger):

- Phrase: "Lamp"

THEN (Action):

- Same URL

- Body:

["0"]

🎤 Final Testing

Voice Commands:

- “Ok Google, Open lamp” → 💡 ON

- “Ok Google, Lamp” → 💡 OFF

🌍 Key Advantage

Unlike Bluetooth:

- Works from anywhere in the world

- Only requires internet connection

🚀 What You Learned

- Internet of Things (IoT) basics

- ESP8266 WiFi communication

- Relay control for real devices

- Mobile app integration (Blynk)

- Voice automation using IFTTT + Google Assistant

💡 Advanced Ideas (Next Level)

- Control multiple devices (use multi-channel relay)

- Add scheduling (auto ON/OFF)

- Add sensors (temperature, motion, etc.)

- Build full smart home system