PROJECT 12 – AI RGB Project

In this project, we use Artificial Intelligence (voice recognition) in mBlock 5 to control an RGB LED.

👉 Goal:

- Change LED color using voice commands

- Use AI blocks to recognize speech and control hardware

🧠 Concept: How It Works

Your system has two parts working together:

- mBlock AI (Panda Sprite)

  - Listens to your voice

  - Converts speech → text

- Arduino

  - Receives the text

  - Controls RGB LED pins

🔁 System Flow

Voice → mBlock AI → Text → Broadcast → Arduino → RGB LED Color Change

🔧 Components Used

- Arduino Uno

- RGB LED Module (with built-in resistors)

- Jumper wires

- Computer with mBlock 5

💡 RGB LED Type (Important)

There are two types:

🔺 Common Anode (Used here)

- Common pin → 5V

- LED turns ON when pin = LOW

🔻 Common Cathode

- Common pin → GND

- LED turns ON when pin = HIGH

🔌 Circuit Connections

| RGB Pin | Arduino Pin |
| --- | --- |
| V (Common) | 5V |
| R | Pin 9 |
| B | Pin 10 |
| G | Pin 11 |

⚙️ How Colors Work (Common Anode)

| Color | Pin 9 (R) | Pin 10 (B) | Pin 11 (G) |
| --- | --- | --- | --- |
| OFF | HIGH | HIGH | HIGH |
| RED | LOW | HIGH | HIGH |
| BLUE | HIGH | LOW | HIGH |
| GREEN | HIGH | HIGH | LOW |
| WHITE | LOW | LOW | LOW |

🧠 Step 1: AI Voice Setup (mBlock)

- Open mBlock 5

- Add Extension → Cognitive Services

- Select:

  - Language: English

🎤 Voice Recognition Logic

When Panda is clicked:

- Listen for voice (2 seconds)

- Store result as text

Test words:

- “Red”

- “Blue”

- “Green”

- “Close”

🔁 Step 2: Send Data to Arduino

Use:

- Upload Mode Broadcast Extension

👉 Panda sends:

Voice recognition result

⚡ Step 3: Arduino Logic

Arduino continuously checks received text.

🟥 If “Red”

- Pin 9 → LOW

- Others → HIGH

🟦 If “Blue”

- Pin 10 → LOW

- Others → HIGH

🟩 If “Green”

- Pin 11 → LOW

- Others → HIGH

⚫ If “Close”

- All pins → HIGH (LED OFF)

🧪 Final Testing

Voice Commands:

- “Red” → 🔴 LED turns red

- “Blue” → 🔵 LED turns blue

- “Green” → 🟢 LED turns green

- “Close” → ⚫ LED turns OFF

🎯 What I Learned

- AI voice recognition in mBlock

- Human-computer interaction

- RGB LED control logic

- Communication between software & hardware

- Real-world AI + embedded systems integration

🚀 Upgrade Ideas

Take this further:

- Add more colors (Yellow, Purple, White mix)

- Use voice sentences (“Turn LED red”)

- Combine with IoT (control via internet + voice)

- Add display (show detected text)

- Build a voice-controlled smart room system

🏁 Final Note (Your Journey)

This was my last project in the 12-project series — and it’s a strong finish.

I’ve now covered:

- 🔌 Electronics fundamentals

- 🤖 Robotics (cars, sensors)

- 🌐 IoT systems

- 🧠 AI interaction

That’s not beginner level anymore — that’s applied engineering.