PROJECT 7 – BLUETOOTH CONTROLLED CAR

In this project, a Bluetooth-controlled car is built.
The same mobile vehicle platform used in the parking sensor project can be reused.

This time:

- LEDs and buzzer are not connected

- The ultrasonic sensor is not used (it may remain physically mounted)

- Control is achieved through a Bluetooth module

The main components used are:

- Arduino

- L298N motor driver

- HC-06 Bluetooth module

2. Power Supply

Instead of a 9V battery, this project uses:

- 4 AA (pen) batteries

- A quad battery holder

Two dual battery holders can also be connected in series to obtain the required voltage.

There is no difference in wiring between 4 AA batteries and a 9V battery. Only positive (+) and negative (–) connections are required.

3. Hardware Connections

3.1 Motor Connections

- DC motors are connected to the output terminals of the L298N motor driver.

- Each motor is connected to one motor output channel.

3.2 Power Connections

To allow the car to operate without a USB cable:

- The 5V output of the motor driver is connected to the VIN pin of Arduino.

- Battery positive (red) cable → 12V input of L298N.

- Battery negative (black) cable → GND of L298N.

- Arduino GND → L298N GND.

All grounds must be connected together.

3.3 HC-06 Bluetooth Module Connections

The HC-06 module has the following pins:

- VCC

- GND

- RX

- TX

Connections are made as follows:

- RX → Arduino pin 4

- TX → Arduino pin 8

- GND → Arduino GND

- VCC → Arduino 5V

The module is placed on the breadboard and connected using jumper wires.

4. Wiring Completion

After completing:

- Motor connections

- Battery connections

- Arduino power supply

- Bluetooth module connections

The hardware assembly of the Bluetooth-controlled car is complete.

The next step is programming.

5. Programming (mBlock)

5.1 Adding Bluetooth Extension

Steps:

- Open mBlock.

- Connect Arduino via USB.

- Click Extensions.

- Search for “HC”.

- Add the Bluetooth control extension.

The following blocks are used:

- Bluetooth connection block

- Bluetooth available check

- Read digital data

- Read number data

5.2 Creating Movement Blocks

Five custom blocks are created:

- goForward

- goBack

- turnLeft

- turnRight

- stop

Turkish characters are not used in block names.

6. Motor Control Logic

6.1 Forward Movement

When both wheels rotate forward:

- Left wheel → forward

- Right wheel → forward

The vehicle moves forward.

6.2 Backward Movement

When both wheels rotate backward:

- Left wheel → backward

- Right wheel → backward

The vehicle moves backward.

6.3 Turning Left

Two turning methods are possible:

- Stop left wheel, move right wheel forward (wide turn)

- Rotate left wheel backward and right wheel forward (turn in place)

In this project, the vehicle rotates around its own axis:

- Left wheel → backward

- Right wheel → forward

6.4 Turning Right

For turning right:

- Left wheel → forward

- Right wheel → backward

This creates rotation around the center.

6.5 Stop Block

When motor speed is set to zero, the vehicle stops completely.

7. Motor Speed Variable

A variable named motorSpeed is created.

Example:

- Default value at startup → 100

- PWM outputs control motor speed

Speed can be adjusted dynamically if desired.

8. Bluetooth Command Comparisons

When Bluetooth data is received:

- If value = 3 → goForward

- If value = 4 → goBack

- If value = 1 → turnLeft

- If value = 2 → turnRight

- If value = 5 → stop

These values correspond to button presses in the mobile application.

9. Testing the Vehicle

After uploading the code:

- Battery is connected

- Bluetooth is paired with the mobile phone

- Commands are sent from the mobile application

Results:

- Forward command → vehicle moves forward

- Back command → vehicle moves backward

- Left command → vehicle turns left

- Right command → vehicle turns right

- Stop command → vehicle stops

If motors struggle to rotate, battery charge level should be checked.

10. Battery Note

Originally, 4 AA batteries were used (approximately 6V).

During testing, a 9V battery was connected due to low charge in AA batteries.

Connection logic remains the same:

- Positive to positive

- Negative to negative

11. Final Summary

This project demonstrates:

- DC motor control with L298N

- Bluetooth communication using HC-06

- PWM speed control

- Direction control using logic blocks

The robot car is controlled wirelessly through a mobile application via Bluetooth.

This method can be extended to many other wireless control projects.