PROJECT 8 – Obstacle Avoiding Robot Car

Obstacle Avoiding Robot Car Project

1. Project Introduction

Hello everyone. In this project, a robot car that avoids obstacles is built.

The robot vehicle moves forward during normal operation. When an obstacle appears in front of the vehicle, it detects the obstacle and changes direction instead of continuing forward.

The robot will:

- Move forward normally

- Detect obstacles using a sensor

- Turn and avoid the obstacle when necessary

2. Robot Platform

The same two-wheeled robot platform used in previous projects can be reused.

This platform already includes:

- DC motors

- Wheels

- Caster wheel

The assembly of these parts was explained in the parking sensor project, so the same platform can be used directly.

3. Components Used

The following components are required for this project.

3.1 Electronic Components

- Arduino Uno

- L298N motor driver

- HC-SR04 ultrasonic distance sensor

3.2 Power Supply

- 2 × 18650 lithium-ion batteries

3.3 Mechanical Components

- Two-wheeled robot chassis

- DC motors

- Wheels

- Caster wheel

4. About the 18650 Battery

The 18650 battery is a rechargeable lithium-ion battery commonly used in electronic devices.

The name 18650 represents the battery dimensions:

- 18 mm diameter

- 65 mm length

- 0 indicates cylindrical shape

These batteries are similar to those used in:

- Mobile phones

- Laptop batteries

Although they look similar to AA batteries, they are slightly larger.

Each battery provides approximately 3.7 volts.

Using two batteries provides:

3.7 V + 3.7 V = 7.4 volts

This voltage is suitable for powering the robot vehicle.

5. Hardware Connections

5.1 Motor Driver Connections

The ENA and ENB jumper clips on the motor driver are removed so that speed can be controlled using PWM.

Connections are made as follows:

- ENA → Arduino pin 9 (PWM)

- ENB → Arduino pin 10 (PWM)

Motor control pins:

- IN1 → Pin 2

- IN2 → Pin 3

- IN3 → Pin 4

- IN4 → Pin 5

These pins control motor direction.

5.2 Powering the Arduino

To allow the robot to operate without a USB cable:

- The 5V output of the motor driver is connected to the VIN pin of the Arduino.

This allows Arduino to receive power from the battery.

5.3 Ultrasonic Distance Sensor Connections

The HC-SR04 ultrasonic sensor has four pins:

- GND

- Echo

- Trig

- VCC

Connections are made as follows:

- GND → Arduino GND

- Trig → Arduino pin 12

- Echo → Arduino pin 13

- VCC → 5V output from motor driver

This sensor measures the distance to obstacles in front of the robot.

5.4 Ground Connections

The GND pins of all components must be connected together:

- Arduino GND

- Motor driver GND

- Sensor GND

This ensures proper circuit operation.

6. Battery and Switch Installation

The batteries are connected using a toggle switch.

This allows the robot to be easily turned ON or OFF.

6.1 Toggle Switch Explanation

Toggle switches are used to control electrical circuits.

Two common types exist:

- 3-pin toggle switches

- 6-pin toggle switches

In this project, the switch operates as an ON/OFF switch.

6.2 Switch Wiring

The positive battery cable is connected to the middle pin of the switch.

Two jumper wires are connected to the left and right pins of the switch and then connected to the positive input of the motor driver.

Operation:

- When the switch is moved left → electricity flows through one side.

- When moved right → electricity flows through the other side.

Both positions allow power to reach the motor driver.

7. Assembly Completion

After completing:

- Motor driver wiring

- Sensor connections

- Battery connections

- Switch installation

The hardware assembly of the obstacle avoiding robot is complete.

The next step is programming.

8. Programming (mBlock)

8.1 Connecting Arduino

Steps:

- Open mBlock 5 software.

- Connect Arduino using the USB cable.

- Click Connect to link Arduino with the software.

Power indicators on the Arduino and motor driver confirm successful connection.

9. Distance Sensor Testing

Before writing the full program, the distance sensor is tested to ensure that it returns correct distance values.

The sensor continuously measures the distance to objects in front of it.

10. Obstacle Detection Logic

The robot's behavior is controlled using an if–then–else condition.

The distance variable is compared with a threshold value.

Example threshold:

20 cm

10.1 When an Obstacle is Detected

If the distance is less than 20 cm, the following actions occur:

- The vehicle stops

- Waits 1 second

- Moves backward for 0.3 seconds

- Turns right for 0.3 seconds

- Continues moving forward

This sequence allows the robot to avoid the obstacle.

10.2 When No Obstacle is Detected

If the distance is greater than 20 cm, the robot continues moving forward normally.

11. Motor Speed Variable

A variable called motorSpeed is created.

Example value:

motorSpeed = 80

This value controls the speed of the motors.

The same speed variable is used for:

- Forward movement

- Backward movement

- Turning movements

If necessary, the speed can be increased later.

12. Program Execution

After uploading the program:

- The distance value appears on the screen

- The robot moves forward when no obstacle is present

When an obstacle is detected:

- The robot stops

- Moves backward briefly

- Turns right

- Continues moving forward

This process repeats automatically whenever an obstacle is detected.

13. Testing the Robot

During testing:

- When the distance is greater than 20 cm → the robot moves forward.

- When an object is placed in front → the robot detects it and avoids it.

The obstacle avoidance behavior can be clearly observed during testing on a table or floor surface.

14. Project Summary

This project demonstrates:

- Ultrasonic distance sensing

- Autonomous obstacle avoidance

- Motor control using L298N

- Decision making using conditional logic

The robot can navigate its environment by detecting obstacles and automatically changing direction.