PROJECT 9 - Line Following Robot Car

1. Project Overview

In this project, we build a robot car that follows a line.

- The robot moves along a black line on a white surface

- It continuously adjusts direction based on sensor input

- Common use: robotics competitions and automation systems

🔹 2. Robot Platform Setup

We reuse the two-wheeled robot chassis, but with a small modification:

- The front of the car is reversed

- Components placement:

  - Arduino Uno → middle

  - Motor driver → slightly forward

  - Battery pack → rear

  - Toggle switch → power control

👉 The distance sensor holder is used only as a visual front reference (not functional here).

🔹 3. Components Used

🧩 Hardware

- Arduino Uno

- L298N Motor Driver

- 2 × DC Motors + Wheels

- Caster Wheel

- 2 × 18650 Batteries

- Toggle Switch

- Line Tracking Sensors (IR-based)

- Jumper wires

- Breadboard

🔹 4. Line Sensor Explanation

⚙️ How It Works

- Uses infrared (IR) reflection

- Detects contrast difference, not color itself

📊 Output Values

- Black surface → 1

- White surface → 0

📦 Types

- 2 sensors (basic)

- 4 sensors (used here)

- 8–32 sensors (advanced precision robots)

🔹 5. Sensor Installation

📍 Mounting Tips

- Place sensors close to ground (~1–1.5 cm)

- Mount under the chassis using:

  - Spacers OR

  - Screws

⚠️ Important

- Insulate sensor base with electrical tape
→ prevents short circuits & false readings

🔹 6. Wiring Connections

🔌 Line Sensor Module → Arduino

| Sensor Module | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| OUT1 | Pin 2 |
| OUT2 | Pin 3 |

🔌 Motor Driver → Arduino

| L298N Pin | Arduino |
| --- | --- |
| ENA | Pin 9 (PWM) |
| IN1 | Pin 4 |
| IN2 | Pin 5 |
| IN3 | Pin 6 |
| IN4 | Pin 7 |
| ENB | Pin 10 (PWM) |

🔋 Power Connections

- Battery → Motor Driver

- Motor Driver → Arduino VIN

- Common GND shared

🔹 7. Testing the Sensors

Before coding logic:

- Check each sensor:

  - Black → 1

  - White → 0

🧠 Variables Created

- leftSensor

- rightSensor

🔹 8. Movement Logic (Core Algorithm)

The robot makes decisions based on two sensors:

🟢 Case 1: On Track

Right = 1 AND Left = 1
→ Move Forward

🔄 Case 2: Drifted Right

Right = 0 AND Left = 1
→ Turn Left

🔄 Case 3: Drifted Left

Right = 1 AND Left = 0
→ Turn Right

🔹 9. Motor Control

🚀 Speed

- Default speed: ~80 PWM

- Can be adjusted for performance

🔧 Movement Functions

- Forward

- Turn Left

- Turn Right

🔹 10. Code Structure (Simplified)

🧱 Step-by-Step Logic

- Initialize variables

- Continuously read sensors

- Apply conditions:

  - Both sensors detect black → forward

  - Left/right mismatch → adjust direction

🔹 11. Testing the Robot

- Use a track with black lines on white background

- Example: figure-8 track

- Observe:

  - Smooth turning

  - Stability on curves

🔹 12. Key Insights

- More sensors = higher accuracy

- Lower height = better detection

- Speed must be tuned:

  - Too fast → misses line

  - Too slow → inefficient

🔹 13. Improvements You Can Try

- Use 4 sensors instead of 2

- Implement PID control (advanced)

- Add speed variation on curves

- Build competition-grade robot

🔹 14. Final Result

✔ Robot successfully follows the line
✔ Adjusts direction automatically
✔ Works continuously in real-time

🚀 Big Picture (For Me Specifically)

This project is actually very important for my path in power + embedded systems:

- Teaches sensor-based control systems

- Builds real-time decision logic

- Introduces feedback systems (control engineering)